Instructor Notes

- We built several of these trainers from used parts for low cost (see picture).

- Students work in pairs to help them learn from each other’s successes and failures.

- Safety is emphasized, since even with low-wattage bulbs (40W), they are dealing with enough current to cause bodily harm.

- Emphasis is placed on estimating expected DMM measurements BEFORE ever attaching the meter. If the estimates are wrong, then students are expected to think about (and correct) their assumptions.

- Students are constantly reminded that the bulbs in their circuit represent “loads.” They could be any industrial loads: motors, solenoids, heaters, lights, etc. The importance isn’t that they understand lights, but that they understand loads.

- Similarly, the different branches of their circuit can be connected/disconnected by means of the switch. In an industrial setting, a branch of the circuit may need to be disconnected using another means. The switch is there for convenience, but it not representative.

- Students are taught to use “ghost” voltage (Fluke 179 meters) for troubleshooting