GS 106						Name: __________________

TOPOGRAPHIC MAP ACTIVITY

Introduction

Maps are fundamental geological tools. We use maps to represent a host of information, including rock distribution, water tables, and, most commonly, the shape of the Earth’s surface. Topography is the shape of the Earth’s surface; maps that represent this surface are called topographic maps. Topographic maps depict the Earth’s surface by means of contour lines, or lines of constant elevation.

Goals – your goals in this lab are twofold. 1) Understand how a topographic map represents the Earth’s surface and be able to create and interpret such a map. 2) Solve problems using published topographic maps and your newly discovered map-reading abilities.

# Materials and Preparation

You will need a pencil (with an eraser) a calculator, and a USGS map of the Howard Butte Quadrangle.

# An Introduction to Topographic Maps

To use topographic maps, you need to understand several basic map features:

Location: Location information identifies the area covered by a map.  United States Geological Survey (USGS) maps cover rectangular areas and are identified by a quadrangle name.  A state index map shows the locations and names of all the quadrangles in a state and can be used to find the quadrangle that covers a particular area.  In addition to quadrangle names, USGS maps are marked with latitude and longitude and other coordinate systems that can be used to locate a map area or feature relative to larger geographic features.  The latitude and longitude of Portland, OR is 45 degrees, 31 min. N and 122 degrees, 41 min. W, respectively.

Map Scale: Maps are scaled-down representations of an area. This means that the distance between two points on a map corresponds to some true distance on the ground. The ratio of map distance to true distance is the map scale. Most people are familiar with a map’s scale bar. A scale bar is a line or bar of some predetermined map length, 2 cm for example, which is labeled with the corresponding true distance, for example, 2 kilometers. Map scales can also be expressed as a ratio or fraction. Many topographic maps, for instance, have a scale of 1:24,000. This means that one cm on the map equals 24,000 cm on the ground, one foot on the map equals 24,000 feet on the ground, etc.  The scale 1:24,000 can also be expressed as a decimal fraction, 1/24000 =0.000041666. While scale bars are handy for a rough approximation of lengths, fractional scales allow precise calculations of lengths. Two equations are particularly useful:

True Distance = (Map Distance) * (Fractional Scale)

Map Distance = (True Distance) / (Fractional Scale)

Note: These expressions assume that map and true distances are expressed in the same units! Thus if you measure map distance in cm and calculate true distance, the calculated distance will be in cm. If true distance is in miles and you calculate map distance, the calculated distance will be in miles.

Orientation: Most maps have a North arrow. When the map is oriented so that the North arrow on the map points to north on the Earth, directions on the map are the same as directions on the ground and the map can be used for navigation. In addition to showing true geographic North, USGS maps show magnetic north.  In California, the discrepancy is about 9o west; compass needles point to a direction that is 9o west of due North.  This “magnetic declination” changes with location—in the western United States, magnetic declinations are east of due North.  Besides North arrows, lines of longitude and latitude or other similar coordinate systems can be used to orient a map.

Contour Lines: Contour lines indicate the shape of the land surface, i.e. its topography. A contour line is the map trace of an imaginary line on the ground that has a particular elevation. One way of visualizing such a line is to imagine a shoreline. Bodies of standing water have level upper surfaces, thus their shorelines are traces on the ground surface of that particular elevation. Topographic maps have many contour lines, each adjacent line differing by a constant elevation difference, the contour interval (Figure 1).

Figure 1: Example of contour lines on a 3-dimensional block diagram.

Contour lines follow some basic rules. If you understand contour lines, you should be able to explain why each of these rules is true:

- The closer spaced contour lines are, the steeper the ground surface is.

- Contour lines never cross

- Contour lines never branch

- Contour lines never change elevation

- Contour lines never skip (e.g., you can never have the sequence of lines 10, 20, 40 without having 10, 20, 30, 40)

- Contour lines always have an up side and a down side (all elevations on one side of the line are higher than the line, all elevations on the other side lower). The up and down side never change along the length of the contour line.

Topographic Map Activity

Howard Butte Quadrangle

1. What is the contour interval of the map?  ___________________________________

Fractional scale? __________________

2. How many minutes of latitude does the entire map represent? ___________________

Longitude?  ___________________

3. What is the elevation of the highest point on the map? _________________

In what corner is it located? ____________________

4. In which direction does Howard Creek flow?  ____________________________

5. Calculate the direct distance from the lookout tower on Howard Butte to Camp Five. Measure the distance in the map in cm ___________.  Multiply it by 24,000 (the fractional scale for this map) Convert this distance to km by moving the decimal point 5 places to the left.

How many kilometers is it from the lookout tower to Camp Five? Show your work

6. What elevation is the Wallowa River at mile 7? ____________________

At mile 14? _________________________

7. Calculate the gradient of the Wallowa River. To do this you:

- Find the difference in elevation between mile 7 and mile 14 of the river.

- Divide this by the distance between the two points. (in this case it is 7 miles).

- Your answer will be in ft/mi. Show your work.

8. At mile 7 on the Wallowa River, which side of the river is the steepest? ___________

9. In which general direction is the Wallowa River flowing?  __________________

10. What is the elevation of Howard Meadow? __________________________________

11. In a short paragraph, characterize the general topography of the map. Where is the highest land? Lowest land? What kind of canyons are the streams in?